# MFB Microscopy Data Organization System — Specification Draft

Version: 0.2 (Draft for Stakeholder Review)
Scope: gjesus3 NAS — Zeiss WSI/Large-Area Microscopy (initial), expandable to other sources
Owner: Jesús Ruiz-Cabello (PI, MFB)
Data Management Lead: Ryan Tasseff (Data Office, CIC biomaGUNE)

## Document Purpose

This specification defines the proposed storage, organization, and governance rules for microscopy data generated by the MFB group. This is a working draft intended to facilitate discussion at the stakeholder alignment meeting.

Throughout this document, items requiring team input are marked:

🔶 DISCUSSION NEEDED: ** *[topic requiring stakeholder input]

These flags identify decisions that cannot be finalized without broader input and should be addressed before or during the alignment meeting.

## 1. Purpose and Guiding Principles

This document establishes the storage, organization, and governance rules for microscopy data generated by the MFB group. Compliance is mandatory for all users granted access to gjesus3.

Core principles:

Raw data is sacred. Original acquisition files are never modified, renamed, or moved after registration.

Traceability is non-negotiable. Every derived output must be traceable back to its raw source(s).

Structure enables collaboration. Consistent conventions allow any group member to find, interpret, and build upon existing work.

Lightweight now, scalable later. The system must be implementable immediately and compatible with future changes in platform workflows and image server (OMERO) integration.

## 2. Directory Structure

The top-level structure on gjesus3 separates immutable raw data from active project work:

/gjesus3/microscopy/ │ ├── raw/ # Authoritative raw acquisitions (READ-ONLY after deposit) │ ├── registry_raw_inventory.csv # Master list of all raw acquisitions │ ├── zeiss-wsi/ # Instrument-specific subfolder │ │ ├── 2025/ # Year of acquisition │ │ │ ├── 2025-01/ # Month of acquisition │ │ │ │ ├── <scan_id>/ # One folder per scan session │ │ │ │ │ ├── *.czi # Raw image file(s) │ │ │ │ │ ├── *.xml # Instrument metadata (if separate) │ │ │ │ │ └── README.txt # Acquisition notes (see Section 4) │ │ │ │ └── ... │ │ │ └── ... │ │ └── ... │ └── other-instruments/ # Future expansion (histology, etc.) │ ├── projects/ # Active project workspaces (READ-WRITE) │ ├── registry_projects.csv # Registered projects │ ├── <project_code>/ # One folder per registered project │ │ ├── _project_metadata.[yaml¦json] # Project metadata file (required) — format TBD │ │ ├── registry_provenance_ledger.csv # Provenance tracking for this project │ │ ├── raw_linked/ # Symlinks to raw data (if automated deposit used) │ │ ├── [subdirectories TBD] # Internal project organization — see discussion below │ │ └── ... │ └── ... │ └── _templates/ # Templates and reference documents ├── project_template/ # Starter structure for new projects ├── _project_metadata.template # Project metadata template (format TBD) └── README_acquisition.txt # Template for acquisition notes

### 2.1 Key Structural Decisions

Registry placement: Rather than a central _registry/ folder, each registry lives at the top of its relevant branch:

raw/registry_raw_inventory.csv

projects/registry_projects.csv

projects/<project_code>/registry_provenance_ledger.csv

🔶 DISCUSSION NEEDED: Project Internal Organization How should projects be subdivided internally? Options include: By researcher (e.g., projects/mfb-lung-study/mbc/, projects/mfb-lung-study/iff/) By study/experiment (e.g., projects/mfb-lung-study/study-01/, projects/mfb-lung-study/study-02/) Flat structure with no required subdivisions (researcher discretion) ISA-style hierarchy (see Section 2.2) The previous draft included inputs/, processing/, and outputs/ subdirectories. These have been removed because: Outputs can become inputs for subsequent processes Enforcing symlinks or path references in these folders adds complexity for non-technical users The provenance ledger already captures input/output relationships Need team input on what level of internal structure is helpful vs. burdensome.

### 2.2 Alignment with Standards: ISA Model Consideration

🔶 DISCUSSION NEEDED: ISA Framework Adoption The ISA (Investigation-Study-Assay) model (ISA specifications) provides a well-established standard for organizing experimental metadata. A potential mapping: ISA Concept; MFB Mapping. Investigation; What we're calling "Project". Study; Potential subdivision within a project. Assay; Roughly equivalent to a raw acquisition/scan session. Advantages of ISA alignment: Established standard with community tools Facilitates data sharing and publication Metadata schemas already defined Compatible with OMERO and other platforms Concerns: ISA has many fields — may be too heavy to start with Terminology shift (Investigation vs. Project) may confuse team Requires JSON or tabular metadata formats (not YAML) Options: Full ISA adoption: Rename "projects" to "investigations," use ISA JSON/CSV formats ISA-lite: Use ISA terminology and structure but only require a subset of fields ISA-compatible: Keep current naming but ensure metadata can be exported to ISA format later Defer: Start with simple structure, migrate to ISA when platform requires it Need team input: Is the overhead of ISA worth the standardization benefits? Which option?

## 3. Naming Conventions

### 3.1 Scan ID (Raw Acquisitions)

🔶 DISCUSSION NEEDED: Scan ID Format The scan ID uniquely identifies each acquisition session. Proposed format: <YYYYMMDD>_<instrument>_<operator>_<sequence>. Component; Description; Example. YYYYMMDD; Date of acquisition; 20250122. instrument; Short instrument code; ZWSI (Zeiss WSI). operator; Operator initials (3 chars); MBC. sequence; Daily sequence number (3 digits); 001, 002. Example: 20250122_ZWSI_MBC_001 Questions for discussion: Is this format suitable for all anticipated instruments? Should sample information be encoded in the ID, or only in metadata? How do we handle multi-day acquisitions or sessions spanning midnight? Is operator-level tracking necessary, or would date + instrument + sequence suffice? Need team input before finalizing.

### 3.2 Operator Codes

Each authorized user is assigned a unique 3-character code:

| Name | Code |
| --- | --- |
| Marta Beraza Cabrerizo | MBC |
| Ainhize Urkola Arsuaga | AUA |
| Irene Fernández Folgueral | IFF |
| Claudia Beatriz Miranda Pérez de Alejo | CBM |
| Itziar Souto Riobo | ISR |
| Ekine Olaizola Bárcena | EOB |
| Tania Orosco Salinas | TOS |

### 3.3 Project Codes

🔶 DISCUSSION NEEDED: Project Code Format and Definition Proposed format: <lab>-<short-description>. Where <lab> is a 3-letter lab code (e.g., MFB for this group). This supports future platform compatibility if multiple labs adopt the system. Example: MFB-lung-fibrosis, MFB-tumor-margins Critical questions: What is a "project"? Aligned with Project Office funding mechanisms (each has an official acronym)? Defined by researcher workflow (may span multiple funding sources)? Equivalent to ISA "Investigation" (a logical grouping of related studies)? Naming source: If funding-aligned: use official project acronyms from Project Office If researcher-defined: allow descriptive short phrases (no spaces, lowercase, hyphens) Granularity: One project per funding grant? One project per research question? One project per publication target? Need team input: How do researchers naturally group their work? Does this align with funding boundaries?

### 3.4 Derived File Naming

🔶 DISCUSSION NEEDED: Derived File Naming Convention Previous draft proposed: <source_scan_id>_<processing_step>_<version>.<ext>. Concerns raised: Multiple inputs: What if a derived file has 3+ source scans? Process granularity: When does changing parameters warrant a new process name? Enforcement: Difficult to validate without tooling Alternative approach: No strict naming convention for derived files Rely entirely on provenance ledger for traceability Recommend readable names but don't enforce format Minimum recommendation: File names should be human-readable and give some indication of content Avoid generic names like output.tiff or final_v2_FINAL.png Include date or version indicator when iterating Need team input: Is a naming convention worth enforcing, or should we rely on provenance tracking?

## 4. Raw Data Deposit Workflow

### 4.1 Overview

The deposit workflow moves data from the acquisition instrument to the permanent raw/ storage on gjesus3.

🔶 DISCUSSION NEEDED: Direct Save vs. Staging Area Options under investigation: Direct network save: Configure instrument to save directly to gjesus3/microscopy/raw/zeiss-wsi/... Requires network configuration and testing Risk: If naming convention not followed at acquisition time, raw folder gets messy Local save + manual transfer: Operator saves to local drive, then copies to gjesus3 Current assumed workflow Risk: Delays, forgotten transfers, inconsistent organization Staging area + automated migration: Save to temporary staging folder with flexible naming, then use a script to migrate to final location Script could enforce metadata collection Script could auto-read instrument metadata Script could create raw folder, project folder, and symlinks More robust but requires script development Need to investigate: Can we set up network save to gjesus3 from the Zeiss workstation? What does the platform want to do with initial raw scans (if anything)? Is script development feasible in the initial rollout? Need IT consultation and platform coordination before finalizing.

### 4.2 Pre-Acquisition

Plan the session. Know which project(s) the data will support before scanning.

Verify access. Confirm you have write access to the appropriate area on gjesus3.

### 4.3 Acquisition

Follow instrument protocols. Use standard Zeiss acquisition settings; record any deviations.

Save to designated location (per workflow option selected above).

### 4.4 Deposit to gjesus3 (Manual Workflow)

This section applies if Option 2 (manual transfer) is used. If automated staging is implemented, this will be replaced by script instructions.

Create the scan folder:

bash mkdir -p /gjesus3/microscopy/raw/zeiss-wsi/2025/2025-01/20250122_ZWSI_MBC_001/

Copy (not move) raw files into the scan folder. Verify copy integrity.

Add acquisition metadata — see Section 4.5.

Generate checksums:

bash cd /gjesus3/microscopy/raw/zeiss-wsi/2025/2025-01/20250122_ZWSI_MBC_001/ sha256sum *.czi > checksums.sha256

Register in the raw inventory — add entry to raw/registry_raw_inventory.csv (see Section 6.1).

Set folder permissions to read-only (Data Management Lead responsibility):

bash chmod -R a-w /gjesus3/microscopy/raw/zeiss-wsi/2025/2025-01/20250122_ZWSI_MBC_001/

Timeline: Registration must be completed same day as acquisition.

### 4.5 Acquisition Metadata (README)

🔶 DISCUSSION NEEDED: Acquisition Metadata Requirements Goal: Capture sufficient metadata at acquisition time to interpret and defend results later. Ideally aligned with REMBI (Recommended Metadata for Biological Images) standard. Questions to resolve: What does the instrument already capture? Need to audit Zeiss .czi metadata fields Anything already in instrument output shouldn't be duplicated in README What must be captured manually? Sample information (type, preparation, staining) — likely not in instrument metadata Experimental context (why this scan, what hypothesis) Deviations from protocol Should README be mandatory? Previous draft: optional but encouraged Proposed change: mandatory with minimum required fields When is metadata collected? At time of scanning (ideal but may slow throughput) Within same-day registration window Via configuration file for automated migration script REMBI alignment: Full REMBI has many fields — which subset is essential for us? Can we start with a minimal set and expand later? Proposed minimum fields (pending REMBI review): Sample type Sample preparation summary Staining/labeling Associated project code Any protocol deviations or quality notes Need to: Audit Zeiss instrument metadata output Review REMBI specification for applicable fields Get team input on what's feasible to collect at acquisition time Templates in Section 7 are placeholders pending this discussion.

## 5. Project Workspace Workflow

### 5.1 Creating a New Project

Register the project by adding an entry to projects/registry_projects.csv.

Create the project folder using the template:

bash cp -r /gjesus3/microscopy/_templates/project_template/ \ /gjesus3/microscopy/projects/<project_code>/

Complete project metadata file with required fields (format TBD — see Section 6.3).

### 5.2 Linking Raw Data to Projects

🔶 DISCUSSION NEEDED: Raw Data Linking Approach Purpose: Enable projects to reference their source raw data without duplicating files. Options: Automated symlinks via deposit script: If staging + script workflow is implemented (Section 4.1 Option 3) Script creates symlinks in projects/<project_code>/raw_linked/ Minimal researcher effort Recommended if automation is feasible Manual symlinks: bash ln -s /gjesus3/microscopy/raw/zeiss-wsi/2025/2025-01/20250122_ZWSI_MBC_001/ \ /gjesus3/microscopy/projects/mfb-lung-study/raw_linked/. Requires command-line comfort Risk of errors, broken links, researcher reluctance Path references only (no symlinks): Just record scan IDs in provenance ledger No actual links in file system Simpler but less convenient for analysis workflows Key question for researchers: How comfortable are you with creating symlinks? Would you use them if they existed, or prefer to navigate to raw/ directly? Would automated linking at deposit time be valuable? Recommendation: If we implement automated deposit (Section 4.1 Option 3), include symlink creation. Otherwise, rely on provenance ledger references only and skip the raw_linked/ folder. Need researcher input on workflow preferences.

### 5.3 Recording Provenance

🔶 DISCUSSION NEEDED: Provenance Recording — Critical for Traceability Purpose: Track how derived outputs were created from raw inputs, enabling trace-back from any publication figure to its source data. Importance: This is essential for publication traceability and reproducibility. However, it is only useful if researchers actually do it. Key challenges: Multiple inputs: A derived file may have many sources: multiple raw scans, parameter files, reference images, etc. Need a format that handles variable numbers of inputs Proposal: separate multiple paths with ; in a single field Multiple outputs: Easier: one row per output, repeating inputs and process description Process description: What level of detail is needed? Options: free text, script name, git repo + commit, ELN entry ID, paper notebook date Need to balance flexibility, machine-readability, and actual traceability Researcher compliance: If this is too burdensome, researchers won't do it If it's not done, raw data tracking becomes impossible Proposed approach: Field; Required; Format; Notes. output_path; Yes; Relative path within project; e.g., analysis/fig1_panel_a.png. input_sources; Yes; Scan IDs and/or file paths, separated by ;; e.g., 20250122_ZWSI_MBC_001;params/config.yaml. process_reference; Yes; Free text — but encourage specificity; e.g., segment_cells.py v1.2, ELN-2025-0042, notebook 2025-01-22 p.15. software; Recommended; Name and version; e.g., ImageJ 1.54f, Python 3.11 + scikit-image 0.21. operator; Yes; Operator code; Who created this output. date_created; Yes; YYYY-MM-DD; When created. notes; Optional; Free text; Any additional context. Questions for discussion: Is this level of detail feasible for routine work? What's the minimum viable provenance record? Should we require provenance only for "publication-quality" outputs, or for everything? Would a simple form/script make this easier than editing CSV directly? Need researcher input on willingness and workflow integration.

## 6. Registry Specifications

### 6.1 Raw Inventory (`raw/registry_raw_inventory.csv`)

The authoritative record of all raw acquisitions.

🔶 DISCUSSION NEEDED: Raw Inventory Fields Goals: Sufficient metadata to find and interpret any raw acquisition Alignment with REMBI where practical Potential alignment with ISA Assay fields (if ISA model adopted) Avoid duplicating what's already in instrument metadata files Current proposed fields:

| Field | Required | Description |
| --- | --- | --- |
| scan_id | Yes | Unique scan identifier |
| date_acquired | Yes | YYYY-MM-DD |
| instrument | Yes | Instrument code (e.g., ZWSI) |
| operator | Yes | Operator code |
| sample_type | Yes | Brief sample description |
| project_code | Recommended | Associated project/investigation (if known at acquisition) |
| study_code | Optional | Associated study within project (if using ISA-style hierarchy) |
| file_count | Yes | Number of raw files |
| total_size_gb | Yes | Total size in GB |
| checksum_file | Yes | Relative path to checksums.sha256 |
| acquisition_metadata_path | Recommended | Path to README.txt or instrument metadata file |
| registered_by | Yes | Who registered the entry |
| registered_date | Yes | When registered (YYYY-MM-DD) |
| notes | Optional | Free-text notes |

Questions for resolution: Which REMBI fields should be in this registry vs. in the acquisition README/instrument files? If adopting ISA, should we include ISA Assay required fields here? (Approximately 3 required fields) What instrument-specific fields (e.g., objective, channels) belong here vs. in referenced metadata? Need to audit Zeiss metadata output before finalizing.

### 6.2 Provenance Ledger (`projects/<project_code>/registry_provenance_ledger.csv`)

Located within each project folder. Tracks derived outputs and their sources.

See Section 5.3 for detailed discussion of fields and challenges.

Proposed fields (pending discussion):

| Field | Required | Description |
| --- | --- | --- |
| output_path | Yes | Relative path to output file within project |
| input_sources | Yes | Scan IDs and/or file paths, ;-separated |
| process_reference | Yes | Script name, ELN ID, notebook reference, etc. |
| software | Recommended | Software and version used |
| operator | Yes | Who created the output |
| date_created | Yes | YYYY-MM-DD |
| notes | Optional | Additional context |

### 6.3 Projects Registry (`projects/registry_projects.csv`)

🔶 DISCUSSION NEEDED: Project Metadata Fields Depends on resolution of: What defines a "project" (funding-aligned vs. researcher-defined) Whether ISA Investigation model is adopted What metadata is needed for platform compatibility

Proposed minimum fields:

| Field | Required | Description |
| --- | --- | --- |
| project_code | Yes | Unique project identifier |
| title | Yes | Full project title |
| lead | Yes | Project lead (operator code) |
| created_date | Yes | YYYY-MM-DD |
| status | Yes | active, archived, completed |
| description | Recommended | Brief description |
| funding_reference | Optional | Link to Project Office grant/acronym if applicable |

Additional fields TBD based on ISA/funding alignment decisions.

## 7. Metadata Templates

🔶 NOTE: Templates are placeholders pending metadata decisions The templates below will need to be revised after the team resolves: REMBI field selection for acquisitions ISA adoption level for projects Project metadata file format (YAML vs. JSON vs. CSV) Current templates are illustrative only.

### 7.1 Acquisition README Template (Placeholder)

================================================================================ ACQUISITION NOTES — [scan_id] ================================================================================ Date: [YYYY-MM-DD] Operator: [Name / Code] Project: [Project code, if known] SAMPLE INFORMATION ------------------ Sample type: [e.g., mouse lung tissue section] Sample ID: [Internal sample identifier] Preparation: [Fixation, embedding, sectioning summary] Staining: [H&E, immunofluorescence markers, etc.] ACQUISITION CONTEXT ------------------- Purpose: [Why this scan was acquired] Protocol ref: [Link to protocol or "standard"] Deviations: [Any deviations from standard protocol] QUALITY NOTES ------------- [Any issues observed: focus problems, artifacts, incomplete coverage, etc.] ================================================================================

### 7.2 Project Metadata Template (Placeholder)

🔶 DISCUSSION NEEDED: Format Decision Option A — YAML (human-readable, familiar to some): yaml project_code: mfb-lung-fibrosis title: "Characterization of fibrotic markers in IPF lung tissue" lead: MBC created_date: 2025-01-22 status: active description: ¦ Analysis of whole-slide images from IPF patient biopsies. team: - MBC - IFF data_sensitivity: standard. Option B — JSON (ISA-compatible, machine-readable): json { "project_code": "mfb-lung-fibrosis", "title": "Characterization of fibrotic markers in IPF lung tissue", "lead": "MBC", "created_date": "2025-01-22", "status": "active", "description": "Analysis of whole-slide images from IPF patient biopsies.", "team": ["MBC", "IFF"], "data_sensitivity": "standard" }. Option C — CSV row (minimal overhead, matches other registries): Project metadata as a single row in registry_projects.csv No separate metadata file needed Less room for extended descriptions Recommendation: If ISA alignment is chosen, use JSON. Otherwise, YAML is more human-friendly for editing. CSV is simplest but least flexible. Need team input on format preference.

## 8. Access Control and Governance

### 8.1 Access Tiers

| Tier | Permissions | Users |
| --- | --- | --- |
| Admin | Full read/write everywhere; manages registries and permissions | Ryan Tasseff, Jesús Ruiz-Cabello |
| Operator | Write to deposit area; read/write to projects/; read-only to registered raw/ | All authorized users |
| Read-only | Read access to specified projects only | TBD (external collaborators if needed) |

### 8.2 Permission Enforcement

Raw data protection is enforced via file system permissions:

After registration, scan folders in raw/ are set to read-only (chmod -R a-w)

Only Admin tier can modify permissions

Operators can create new scan folders but cannot modify registered ones

### 8.3 Compliance Requirements

Access to gjesus3/microscopy is conditional on agreement to:

Follow all naming conventions exactly as specified.

Register all raw acquisitions in the raw inventory same day as acquisition.

Never modify, rename, or delete files in raw/ after registration.

Record provenance for derived outputs intended for publication or sharing.

Report issues immediately to the Data Management Lead.

### 8.4 Violation Handling

First violation: Written reminder and correction of the issue.

Repeated violations: Escalation to PI; potential temporary suspension of write access.

Serious violations (e.g., deletion of raw data): Escalation to PI; potential permanent access revocation.

### 8.5 Onboarding Process

New users must:

Read this document in full.

Attend orientation session with the Data Management Lead.

Complete a supervised test deposit.

Acknowledge compliance requirements in writing.

## 9. Data Protection Considerations

### 9.1 Standard Data

Most microscopy data (animal tissue, cell lines, phantoms) is standard sensitivity and requires no special handling beyond the conventions in this document.

### 9.2 Restricted Data (Human/Clinical)

Data derived from human subjects or clinical samples requires additional safeguards:

Mark the project as data_sensitivity: restricted in project metadata.

No patient identifiers in file names, folder names, or unencrypted metadata.

Access limited to named individuals approved for the study.

Tania Orosco Salinas serves as safeguard/advisor for clinical data handling.

Consult institutional policies and ethics approvals before data deposit.

## 10. Integrity and Backup

### 10.1 Checksums

All raw acquisitions must have SHA-256 checksums generated at deposit time. Periodic verification (quarterly recommended) ensures data integrity.

bash # Generate at deposit: sha256sum *.czi > checksums.sha256 # Verify later: sha256sum -c checksums.sha256

### 10.2 Backup Status

🔶 DISCUSSION NEEDED: Backup Strategy — Requires PI Decision Current state: gjesus3 is RAID-protected but not backed up by IT. RAID protects against single-drive failure but not against: Multiple drive failures Accidental deletion Ransomware/corruption Physical disaster (fire, flood) Questions for Jesús: Do we want offsite backup? If yes, what scope: Everything in /gjesus3/microscopy/? Only raw/ (irreplaceable original data)? Only "critical projects" (designated by PI)? Who is responsible for backup? IT (requires negotiation/budget)? Lab-managed solution (external drives, cloud)? Project-lead responsibility for their own projects? What is acceptable risk? If gjesus3 had a catastrophic failure today, what would the impact be? How much data loss is tolerable? Current mitigation (without dedicated backup): Checksums catch silent corruption Original files may exist on acquisition workstation (short-term) Critical projects could maintain manual copies (external drive, cloud) Need PI decision before finalizing backup approach.

## 11. OMERO Compatibility

This system is designed for future integration with OMERO (Open Microscopy Environment):

| MFB System Element | OMERO Mapping |
| --- | --- |
| Scan ID | Dataset name |
| Project code | Project name |
| Raw inventory metadata | Key-value annotations |
| Provenance records | ROI annotations, attachments |

When OMERO is deployed, a migration script can ingest registered scans while preserving metadata and relationships. The ISA-style hierarchy (if adopted) maps naturally to OMERO's Project/Dataset structure.

## 12. Quick Reference Checklists

### After Every Acquisition (Same Day)

☐ Create scan folder with correct naming: YYYYMMDD_INST_OPR_###

☐ Copy all raw files to the scan folder

☐ Generate checksums: sha256sum *.czi > checksums.sha256

☐ Complete README.txt with required acquisition metadata

☐ Add entry to raw/registry_raw_inventory.csv

☐ Notify Data Management Lead to set folder read-only

### Starting a New Project

☐ Add entry to projects/registry_projects.csv

☐ Copy project template to new folder

☐ Complete project metadata file

☐ (If automated) Link relevant raw data in raw_linked/

### Before Publication

☐ Verify all figures have provenance ledger entries

☐ Confirm raw data checksums still valid

☐ Document software versions in provenance records

☐ Update project status to completed

## 13. Implementation Roadmap

🔶 NOTE: Sequencing TBD based on discussion outcomes

| Phase | Activities | Dependencies |
| --- | --- | --- |
| Phase 0: Decisions | Resolve all 🔶 discussion items | Stakeholder meeting |
| Phase 1: Foundation | Finalize naming conventions, create directory structure, deploy templates | Phase 0 |
| Phase 2: Pilot | Onboard 2-3 users, test deposit workflow, refine based on feedback | Phase 1 |
| Phase 3: Rollout | Onboard remaining users, document lessons learned | Phase 2 |
| Phase 4: Automation | Develop deposit script (if feasible), streamline workflows | Phase 3 |
| Phase 5: Platform Alignment | Coordinate with Irantzu on platform integration, prepare for OMERO | Ongoing |

## 14. Open Questions Summary

For convenience, all discussion items collected in one place:

| # | Topic | Section | Key Question |
| --- | --- | --- | --- |
| 1 | Project internal organization | 2.1 | By researcher, by study, flat, or ISA-style? |
| 2 | ISA framework adoption | 2.2 | Full ISA, ISA-lite, ISA-compatible, or defer? |
| 3 | Scan ID format | 3.1 | Is proposed format suitable? What about edge cases? |
| 4 | Project definition | 3.3 | Funding-aligned or researcher-defined? |
| 5 | Derived file naming | 3.4 | Enforce convention or rely on provenance ledger? |
| 6 | Deposit workflow | 4.1 | Direct save, manual transfer, or automated staging? |
| 7 | Acquisition metadata | 4.5 | Which REMBI fields? Mandatory README? |
| 8 | Raw data linking | 5.2 | Automated symlinks, manual symlinks, or paths only? |
| 9 | Provenance recording | 5.3 | Feasibility? Minimum viable record? Required when? |
| 10 | Raw inventory fields | 6.1 | REMBI/ISA alignment? Instrument-specific fields? |
| 11 | Project metadata format | 7.2 | YAML, JSON, or CSV? |
| 12 | Backup strategy | 10.2 | Scope, responsibility, acceptable risk? |

## 15. Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 2025-01-22 | R. Tasseff (draft) | Initial draft |
| 0.2 | 2025-01-22 | R. Tasseff | Restructured registries; added discussion flags; incorporated feedback on project structure, provenance, symlinks, metadata standards |

## 16. Appendices

### Appendix A: Instrument Codes

| Code | Instrument | Location |
| --- | --- | --- |
| ZWSI | Zeiss Axiocam 7 (WSI/large-area) | MFB Lab |
| TBD | Additional instruments to be added | — |

### Appendix B: File Format Reference

| Format | Extension | Typical Source | Notes |
| --- | --- | --- | --- |
| Carl Zeiss Image | .czi | Zeiss microscopes | Native format; preserves all metadata |
| OME-TIFF | .ome.tif | Various / export | Open standard; good for interoperability |
| TIFF | .tif, .tiff | Various | Common export; may lose metadata |
| JPEG | .jpg | Screenshots | Lossy; not for quantitative work |

### Appendix C: Reference Standards

| Standard | Relevance | Link |
| --- | --- | --- |
| ISA | Investigation-Study-Assay metadata model | isa-specs.readthedocs.io |
| REMBI | Recommended Metadata for Biological Images | Nature Methods paper |
| OME | Open Microscopy Environment data model | openmicroscopy.org |

### Appendix D: Glossary

| Term | Definition |
| --- | --- |
| Raw data | Original files as output by the instrument, unmodified |
| Derived data | Any file created by processing raw data |
| Scan ID | Unique identifier for a single acquisition session |
| Provenance | Documented history of where data came from and how it was processed |
| WSI | Whole Slide Imaging |
| ISA | Investigation-Study-Assay (metadata standard) |
| REMBI | Recommended Metadata for Biological Images |

End of document